BPA - British Pyrotechnists Association

Display Operators Certificate

Examination Specimen Paper (L1 Exam)

# Instructions

# This paper consists of three parts

Part 1 (Foundation)    30 Multiple choice items    1 Hour

(Questions 1 to 30)

Part 2 (Intermediate)  15 Multiple choice items    1/2 Hour

(Questions 31 - 45)

Part 3 (Advanced)      1 Extended Question          1/2 Hour

Foundation level candidates      Attempt questions 1 to 30 only    1 Hour

Intermediate level candidates    Attempt questions 1 to 45 only    1 1/2 Hours

Advanced level candidates       Attempt all questions                    2 Hours

Multiple choice questions (1 to 45)

Select ONE response [A, B, C or D] which you consider to be the correct answer in each case.

Attempt all questions (do not leave blanks)

No marks are rewarded for incorrect answers or omissions.

___________________________________________________________________________________

Section 1

Level 1 (Foundation)

Attempt all 30 questions

1.    An 'explosive' substance is one which is best described as?

A    Something used to make a firework

B    A substance which makes a loud noise when ignited

C    A substance which produces energy very rapidly when ignited

D    A substance which needs to be handled very carefully

Ref: section 1.1(a)

2.    'Black powder' may be found in each of the following except?

A    Paper sleeved quickmatch (eg 'Spanish match')

B    The lifting charge of a shell

C    ICI 'slow' (green) igniter fuse (PIC)

D    The composition of a rocket motor

Ref: section 1.1(a), 1.1(c), 1.2(b), 1.2(d)

3.    According to the British Standard BS7114, relating to fireworks,

which of the following is correct?

A    'Category 3' fireworks cannot be used by members of the public

B    If a firework is to be used in a domestic garden, it must belong to 'Category 2'

C    'Category 4' fireworks are for professional use only

D    Roman candles always belong to 'Category 3'

Ref: section 1.1(b)

4.    The familiar ICI 'plastic igniter cord' (PIC) is available in a variety of colours, Which of the

following burns at the slowest speed (ie has the lowest burn rate)?

A    Green

B    Blue (PW6)

C    Brown

D    All burn at approximately the same speed

Ref: section 1.1(c)

5.    Which of the following types of fuse burns almost as rapidly as 'tapematch'?

A    Piped (or sleeved') blackmatch

B    Green PIC

C    Chinese green braided fuse

D    A pressed delay fuse

Ref: section 1.1(c)

6.    The diagram above shows a typical spherical aerial shell. The Part of the shell labelled 'X'

contains?

A    The lifting charge

B    The 'stars' or units

C    The slow ignition fuse

D    The Bursting charge

Ref: section 1.2(b)

7.    Which of the following statements about a typical aerial shell is incorrect?

A    The calibre of the shell is normally described by its diameter

B    The quickmatch leader fuse normally goes into the lifting charge

C    A time delay fuse separates the star chamber bursting charge from the lifting charge

D    The quickmatch fuse burns very slowly

Ref: section 1.2(b)

8.    Which of the following malfunctions (failures) will cause a shell to leave the mortar effectively but

not 'burst'  (eject the stars) in the normal way?

A    The quickmatch fuse does not burn fully and extinguishes

B    The internal time delay fuse fails to ignite

C    Moisture in the mortar tube causes the lifting charge to get wet and fail to provide full power

D    If the shell is a very 'tight fit' in the mortar tube

Ref: section 1.2(b)

9.    In relation to visual shell performance, another word or term for 'pistil' is?

A    Colour change

B    Kamuro

C    Heart  (or 'core')

D    Strobe

Ref: section 1.2(b)

10.    Which of the following best describes the performance of a mortar loaded mine?

A    An instantaneous ejection of stars and/or noise effects from ground level to a good height

B    A pyrotechnic device which is placed in the ground and subsequently explodes

C    A device similar in performance to a shell

D    A device similar in performance to a 45mm single shot comet Roman candle

Ref: section 1.2(c)

11.    Which of the following statements about sky rockets is incorrect?

A    Rockets tend to fly into wind when launched

B    The stability of a rocket is affected by the length and weight of its stick

C    The 'Flying Pigeon' is a special type of rocket which is designed to fly horizontally along a

rope

D    Rockets are always safer to use in displays than shells

Ref: section 1.2(d), 1.3(a)

12.    Which of the following statements about Roman candles is correct?

A    The 'calibre' of a roman candle is always described by its length

B    Roman candles are normally designed to produce a series of stars and/or noise effects at

regular intervals

C    The 'bouquet' of Roman candles always contains three candles in a 'fan' formation

D    Roman candles always produce at least eight 'shots'

Ref: section 1.2(e)

13.    A 'multishot battery' (cake) performs in much the same way as?

A    A series of single shot Roman candles connected by a delay fuse

B    A bundle of multishot Roman candles fused to fire together

C    A repeating mine Roman candle

D    None of the above

Ref: section 1.2(e), 1.2(f)

14.    From time to time a multishot battery (cake) will 'explode' when firing with the result that it’s

Internal tubes are separated, which of the following could be responsible for this?

A    The cake is not sufficiently well secured (eg by staking or burying)

B    The cake does not meet the British Standard BS7114:2:88

C    The cake is not heavy enough

D    A Unit (for example, a maroon unit) blows prematurely without being discharged from its

tube

Ref: section 1.2(e), 1.3(b)

15.    Which one of the following statements about multishot batteries (cakes) is incorrect?

A    They may be designed to fire over 100 projectiles (units)

B    They do not need to be stabilised by staking, burying or sand bagging

C    They are supplied in a variety of calibres

D    They can be fired electrically or manually

Ref: section 1.2(e)

16.    Which of the following statements about fountains (gerbes) is incorrect?

A    They are often used to provide effects in static set pieces

B    They can often be manufactured to provide high thrust to turn wheel devices

C    They present no hazard to spectators and operators

D    They are available in a variety of 'calibres' often indicated by diameter or power weight

Ref: section 1.2(g)

17.    Which of the following is not normally a type of  'lancework' device?

A    A 'GOOD NIGHT' set piece

B    A 'Waterfall'

C    A 'Battle scene'

D    A set piece depicting a portrait

Ref: section 1.2(f)

18.    When planning a small public display (a November school event, for example) the site should

always?

A    Be at least 1 mile from the nearest hospital

B    Be large enough to allow a bonfire

C    Allow for spectators to enter and exit relatively easily

D    Have a large 'grassed' area for the fireworks to be set up

Ref: section 1.3(a)

19.    When firing a display manually, which of the following is not sensible advice?

A    Always wear a hard hat or some form of head protection

B    Wear overalls which have an exposed pocket in which to keep unused portfires

C    Ensure adequate eye protection

D    Allocate specific duties to each operator and follow a firing order if possible

Ref: section 1.3(a)

20.    Which of the following should always be available to operators on site?

A    An adequate first aid kit

B    A mobile telephone

C    Adequate refreshments

D    A good supply of portfires

Ref: section 1.3(a)

21.    When laying out a display site, which of the following arrangements is normally used with regard

to safety distance, the first named firework is closest to the crowd line?

A    Shells, Rockets, Multishot Batteries, Roman Candles

B    Roman Candles, Multishot Batteries, Set Pieces, Shells

C    Set Pieces, Roman Candles, Multishot Batteries, Shells

D    Roman Candles, Set Pieces, Multishot Batteries, Shells

Ref: section 1.3(a)

22.    Which of the following pair of fireworks types present approximately the degree of hazard and

risk to spectators?

A    Shells and mines

B    30mm bombette Roman candles and multishot cake of the same calibre

C    Set pieces and roman candles

D    Rockets and roman candles

Ref: section 1.3(a)

23.    Several materials can be used for manufacturing mortar tubes, which of the following is generally

considered to be the safest and most reliable?

A    Glass fibre composite ('fibreglass')

B    Paper (cardboard)

C    Steel

D    High density polythene (HDPE)

Ref: section 1.3(b)

24.    An operator places a 100mm shell, by mistake, into a 125mm mortar tube. What is the likely

result when the shell is fired?

A    The shell will explode immediately in the tube

B    The shell will perform exactly as expected with no adverse result

C    The shell may leave the tube, but will burst at a much lower height

D    The shell will leave the tube at a much higher speed than normal

Ref: section 1.2(b), 1.3(b)

25.    A large (250mm) shell is to be fired from a paper (card) mortar tube. Which of the following is

not advisable?

A    The tube should be buried to at least one third of its length

B    The shell should be lowered into the tube using a string or rope attachment

C    The shell can be fired from a wooden 'rack' consisting of four individual tubes containing

shells to be fired in sequence

D    The shell can be fired electrically

Ref: section 1.3(b)

26.    Which of the following statements about aerial shells is incorrect?

A    Shells belong to 'Category 4'  as defined by the British Standard BS7114

B    Shells should always be fired from mortar tubes with an internal diameter slightly larger than

the diameter of the shell

C    Shells generally present a greater hazard (and risk) to operators than Roman candles

D    Shells always present a lower risk (and hazard) to operators than mines of the same calibre

Ref: section 1.2(b), 1.2(c), 1.1(b), 1.3(c)

27.    A simple risk assessment for a routine firework display (for example, to be staged on a school

playing field) would consider all of the following points except?

A    How near the vehicle carrying the fireworks can be parked to the site

B    The size of 'fall out' zone available

C    The types (and calibre) of fireworks to be used in the display

D    The location of neighbouring houses, which are outside the school grounds

Ref: section 1.3(a), 1.3(c)

28.    The main purpose of a risk assessment is to?

A    Identify the hazards, groups at risk and ensure adequate precautions are in place

B    Comply with insurance requirements

C    Ensure the most spectacular display

D    Eliminate all hazards and risks from the display

Ref: section 1.3(c)

29.    When preparing a lancework set pieces, there are several options available for fusing together the

individual lances. Which of the following type of fuse  presents the greatest risk of premature

ignition of the set piece during a display?

A    Tape match

B    'Brown' PIC ('fast igniter')

C    Piped blackmatch (Spanish match)

D    German type fast igniter cord

Ref: section 1.1(c), 1.3(c)

30.    Which of the following statements is correct?

A    The length of mortar tube used to fire a 75mm shell can vary

B    Display mines are designed to fire a series of stars at regular intervals

C    A maroon (salute) shell presents a mush lower hazard to operators than a chrysanthemum

shell

D    The function of the 'bursting charge' in a shell is to eject the shell from the mortar tube when a

gas is formed

Ref: section 1.2(b), 1.2(c), 1.3(c)

End of section 1

Now go back and check your work

___________________________________________________________________________________

Section II

Level 2 (Intermediate)

Attempt all 15 questions

31.    Ten 75mm aerial shells, in a rack, are to be fused together so that they fire in sequence, at

approximately 3 second intervals, during a display. Which of the following is incorrect?

A    The leader fuses of the shells could be connected together using 'green' PIC at roughly 7.5cm

intervals, in a chain

B    The leader fuses of the shell could be connected together using pressed powder delays, in a

chain

C    The leader fuses could be connected together using piped blackmatch

D    The chain sequence could be initiated by one electrical igniter

Ref: section 2.1(a), 2.1(b)

32.    A multishot battery (cake) is to be fired using and electrical igniter. Which of the following is a

reliable 'sequence' of fusing?

A    Electrical Igniter - Slow PIC - Lift powder in tube

B    Electrical Igniter - Piped Match - Slow PIC - Lift powder in tube

C    Electrical Igniter - Fast PIC - Lift powder in tube

D    Electrical Igniter - Chinese Slow Fuse - Lift powder in tube

Ref: section 1.1(c), 2.1(b)

33.    Two identical electrical igniters are connected in series to a power supply. Which of the following

statements is incorrect?

A    The igniters carry the same current

B    The igniters should, under normal circumstances, 'fire' simultaneously

C    The igniters will fire only  if the voltage of the power supply is high enough (ie exceeds a

minimum value)

D    If a third igniter is placed in series, the current will be lower if the same power supply is used

Ref: section 2.1(b)

34.    Which of the following is not the case when comparing electrical and manual firing methods?

A    The injury risk to operators is generally lower in the case of electrical firing

B    A greater degree of control can be achieved using electrical firing

C    Electrically fired displays are generally quicker to set up than manually fired displays

D    Electrical firing is normally preferred for pyromusical shows

Ref: section 2.1(b)

35.    Which of the following statements concerning aerial shells is incorrect?

A    Willow or Kamuro shells always present a lower hazard and risk to spectators than

chrysanthemum shells of the same

diameter, fired under identical conditions

B    Large cylindrical multibreak shells may require a mortar tube of greater length than a single

break shell of the same calibre

C    It is acceptable to insert an electrical igniter directly into the lifting charge of a shell

D    'Ring' shells may appear as either a line, ellipse or circle depending on the angle of viewing

Ref: section 2.2(a)

36.    In order to achieve maximum horizontal range, a mortar tube used for firing aquatic shells should

be orientated at which of the following angles, measured relative to the horizontal?

A    20 Degrees

B    30 Degrees

C    45 Degrees

D    60 Degrees

Ref: section 2.2(a)

37.    Which of the following statements, concerning the use and performance of  Roman candles, is not

true?

A    They may produce crossette effects

B    They may produce repeating mine effects

C    It is common to combine candles of different calibre on a single frame to achieve varying

visual effects

D    They are never used on set piece devices

Ref: section 2.2(b)

38.    When firing a display from a Roof top, which of the following is the least important

consideration?

A    The height of the building

B    The material from which the roof is made

C    The type of fireworks to be included in the display

D    The duration of the display

Ref: section 2.3

39.    A comprehensive risk assessment for a display on a particular site would not include which of the

following?

A    An assessment of weather rockets are suitable

B    A recommendation regarding maximum shell calibre to be used

C    The most convenient route to the site from base

D    A consideration of crowd distribution around the site

Ref: section 2.3

40.    According to the UN system of classification of explosives, which of the following is incorrect?

A    Many fireworks are designated by UN number 0336

B    All explosives are designated as 'Class 1'

C    Division 1.1 indicates a minor explosion hazard

D    Commercial fireworks mostly belong to compatibility group G

Ref: section 2.4(a)

41.    A carton is marked with UN designation 1.3G in a hazard diamond. This would indicate that the

contents present?

A    A mass explosion hazard

B    A minor blast or projection hazard

C    No significant hazard

D    No hazard at all, because they are insensitive

Ref: section 2.4(a)

42.    The Carriage of Explosives by Road Regulations deal with a number of requirements except?

A    Limits on the quantities of fireworks which can be carried

B     Requirements for the training of drivers

C     Requirements that the firework should meet the appropriate British Standard

D    Construction of vehicles and containers used for transport

Ref: section 2.4(b)

43.    Which of the following statements relating to a TREM card is incorrect?

A    It should carry relevant information about the nature and hazard of the load

B    It should be kept in the load compartment during the journey

C    It is primarily intended for the emergency services, if required

D    It should be discarded when the journey is complete and the load is removed or transferred

Ref: section 2.4(b)

44.    A box containing fireworks of an unknown type is labelled UN 0335 1.3G The contents could be?

A    Aerial Shells

B    Sparklers

C    'Category 2' Roman Candles

D    Electrical Igniters

Ref: section 1.1(b), 2.4(a)

45.    In connection with typical multishot batteries ('cakes') which of the following is incorrect?

A    They normally consist of numerous individual tubes connected by a delay fuse

B    They all come within 'Category 4' of BS7114

C    They may contain both mine and bombette units

D    They may be allocated hazard division 1.3G

Ref: section 1.2(f), 2.4(b)

End of Section II

Now go back and check your work

__________________________________________________________________________________

Section III

Level 3

Attempt this question

(30 minutes)

46.    You are asked to design a five minute firework display suitable for 'daylight' conditions.

Discuss the various firework types and effects that would be suitable for such a display.

Your answer should include a brief survey of the firework types from the point of view

of a spectator safety, suitability and visual performance.

( 15 marks )

Ref: section 3.1, 3.2